# Signal Standards Framework for Emerging Analog AI Computing

A Cross-Technology Synthesis for Industry Standardization

December 2025 | Working Draft v1.0

# Executive Summary

The semiconductor industry stands at a pivotal juncture, transitioning from the rigid determinism of von Neumann architectures to the probabilistic efficiency of Analog In-Memory Computing (AIMC). This paradigm shift, driven by the exponential growth of AI workloads and the fundamental limitations of the "memory wall," necessitates a comprehensive re-evaluation of the industrial framework governing computing hardware.

This document synthesizes research across multiple analog memory technologies—including Resistive RAM (ReRAM), Phase-Change Memory (PCM), Ferroelectric FETs (FeFET), Flash/SONOS, and Magnetic Tunnel Junctions (MTJ)—to identify

common ground for establishing signal standards in the emerging analog behavior markets. The analysis draws from the International Roadmap for Devices and Systems (IRDS), NIST metrology initiatives, IEEE working groups, ISO/IEC standardization efforts, and cutting-edge industrial research.

The central finding is that while the underlying physics of each technology differs substantially, they share fundamental challenges and requirements that provide a foundation for unified standardization: the encoding of neural weights as continuous physical properties, the management of inherent stochasticity, the need for drift compensation, and the requirement for interoperability with digital systems.

# Table of Contents

# 1. The Common Ground: Universal Principles Across Technologies

## 1.1 The Fundamental Analog Computation Paradigm

All analog AI technologies share a common computational paradigm that distinguishes them from digital systems. This paradigm is built on two fundamental physical laws:

- Ohm's Law for Multiplication: I = V × G — The synaptic weight encoded as conductance (G) is multiplied by the input activation encoded as voltage (V) to produce a current (I)

- Kirchhoff's Current Law for Accumulation: I_total = Σ(V_i × G_i) — Currents from multiple devices sum naturally on a shared bitline, performing the accumulation step of matrix-vector multiplication

This physics-based computation enables massive parallelism—an entire matrix-vector multiplication in a single clock cycle—with potential energy efficiency improvements of 100× over digital accelerators.

## 1.2 Universal State Variables and Encoding Mechanisms

Despite their different underlying physics, all analog memory technologies encode weights as continuous physical state variables:

| Technology | State Variable | Physical Mechanism |
| --- | --- | --- |
| ReRAM/RRAM | Filament conductance | Oxygen vacancy migration |
| PCM | Crystalline-amorphous ratio | Chalcogenide phase transition |
| FeFET | Remnant polarization (P_r) | Ferroelectric domain switching |
| Flash/SONOS | Trapped charge (Q) | Fowler-Nordheim tunneling |
| MTJ/MRAM | Magnetization orientation | Spin-transfer/orbit torque |

The critical insight is that regardless of the underlying mechanism, all technologies must address the same fundamental challenge:

translating a continuous physical state into a usable analog weight value while managing noise, drift, and variability.

## 1.3 Universal Challenges Requiring Standardization

The research reveals five universal challenges that span all analog AI technologies and form the basis for signal standards:

- Stochasticity: All technologies exhibit inherent randomness—from ion migration in ReRAM to nucleation in PCM to domain switching in FeFET. Standards must define acceptable statistical distributions.

- Temporal Drift: Conductance states are not static. PCM exhibits power-law drift (G(t) = G₀(t/t₀)^−ν), while FeFET suffers from depolarization. Standards must specify drift budgets and compensation protocols.

- Read/Write Asymmetry: SET and RESET operations differ in speed, energy, and precision across all technologies, complicating symmetric weight updates required for training.

- Noise Floor: Random Telegraph Noise (RTN), thermal noise, and shot noise limit effective bit precision. Standards must define signal-to-noise ratio (SNR) requirements.

- Digital Interface: All analog systems require conversion to/from digital domains. Standards must specify DAC/ADC precision, encoding schemes (PWM vs. PAM), and protocol interfaces.

# 2. Proposed Signal Standards Framework

## 2.1 Weight State Specification Standard

The foundational standard must define how analog weight states are specified, measured, and communicated. Based on the synthesis of research, we propose the following framework:

### 2.1.1 Conductance State Representation

Standardize weight representation as normalized conductance: G_norm = (G − G_min) / (G_max − G_min), where G_min and G_max are technology-specific bounds.

- Dynamic Range: Specify minimum on/off ratio requirements (e.g., >10³ for inference, >10⁴ for training)

- Precision Tiers: Define "Inference Grade" (4-6 effective bits), "Edge Training Grade" (6-8 bits), and "Datacenter Grade" (8+ bits)

- Signed Weight Encoding: Mandate differential encoding (W = G⁺ − G⁻) as the standard method for representing signed weights

### 2.1.2 Statistical State Specification

Move from deterministic to probabilistic specifications:

- Target Conductance (G_T): The intended weight value

- State Variance (σ_G): The acceptable standard deviation around G_T

- Confidence Interval: The probability that the actual conductance falls within the specified bounds (e.g., 99.7% within 3σ)

## 2.2 Temporal Stability Standards

The research unanimously identifies temporal drift as a critical challenge. A unified standard must address:

### 2.2.1 Drift Characterization Protocol

- Drift Coefficient (ν): Standardize measurement of the power-law drift exponent across temperature ranges

- Drift Variance (σ_ν): Characterize device-to-device and state-to-state variation in drift behavior

- Retention Classes: Define tiered retention requirements:

- Class A: <1% conductance change over 1 year at 85°C (enterprise)

- Class B: <5% change over 1 year at 85°C (consumer)

- Class C: <10% change over 1 month at 85°C (edge/IoT)

### 2.2.2 Drift Compensation Interface

Standardize the interface for drift compensation systems:

- Reference Column Protocol: Define placement and calibration of fixed-reference conductance columns for global drift tracking

- Compensation Signal Format: Specify the digital interface for communicating drift corrections to ADC/gain stages

- Recalibration Trigger: Define conditions that mandate weight refresh (e.g., accuracy degradation threshold)

## 2.3 Input/Output Encoding Standards

### 2.3.1 Input Activation Encoding

Standardize two primary modulation schemes with defined trade-offs:

| Scheme | Advantages | Standardization Focus |
| --- | --- | --- |
| Pulse Amplitude (PAM) | Single-cycle operation; maximum throughput | DAC linearity; IR-drop compensation; voltage headroom |
| Pulse Width (PWM) | Inherent linearity; noise robustness; device-agnostic | Timing precision; integration window; bit-serial protocols |

### 2.3.2 Output Sensing Standards

- ADC Precision Requirements: Define minimum effective number of bits (ENOB) for different precision tiers

- Sensing Mode: Standardize current-sensing vs. voltage-sensing architectures with defined conversion formulas

- Integration Time: Specify relationship between integration window and noise floor for charge-domain computation

# 3. Metrology Standards for Analog AI

## 3.1 From Geometric to Functional Metrology

The 2024 IRDS Metrology Chapter identifies a critical inflection point: for analog AI devices, the focus must expand from measuring

structure (critical dimensions, overlay) to measuring function (conductance distributions, noise spectra, drift dynamics).

### 3.1.1 Statistical Metrology Protocols

Standardize measurement protocols that capture full statistical behavior:

- Conductance PDF Measurement: Define sampling size, temperature conditions, and reporting format for conductance probability density functions

- Noise Power Spectral Density: Standardize measurement bandwidth (e.g., 1Hz–1MHz) and integration time for RTN and 1/f noise characterization

- Endurance Cycling: Define "Analog Endurance" as the number of cycles before σ_G exceeds specified threshold (distinct from binary Pass/Fail)

### 3.1.2 NIST-Aligned Measurement Standards

Align with NIST's neuromorphic device measurement program:

- Synthetic Cortical Function Metrics: Develop chip-scale functional measurements analogous to fMRI—visualizing information flow across neuromorphic arrays

- Attajoule-Scale Energy Measurement: Standardize protocols for measuring switching energies in the 1–100 aJ range required for superconducting and spintronic devices

- Stochastic Verification: For probabilistic devices (e.g., superparamagnetic MTJs), verify that switching distributions match expected Boltzmann statistics

## 3.2 Benchmarking Standards

Current MLPerf benchmarks disadvantage analog AI by requiring strict accuracy thresholds. Adopt NeuroBench-style metrics:

- Accuracy-per-Watt: Primary efficiency metric that captures the energy-accuracy trade-off

- Synaptic Operations (SynOps): Event-based throughput metric for spiking neural networks

- Footprint: Memory utilization metric that highlights sparse connectivity advantages

# 4. Digital-Analog Interface Standards

## 4.1 Chiplet Interconnect (UCIe Integration)

Analog AI accelerators are increasingly deployed as chiplets alongside digital processors. Standardization must build upon Universal Chiplet Interconnect Express (UCIe):

- Heterogeneous Node Support: Enable analog chiplets (fabricated on 40nm+ nodes optimized for NVM) to interface with leading-edge digital hosts (3nm)

- Protocol Modes: Define CXL-compatible memory-mapped mode and streaming mode for neural network inference pipelines

- Calibration Channel: Add standard sideband channel for drift compensation data and health monitoring

## 4.2 Programming Interface Standards

### 4.2.1 Weight Programming Command Set

Define a standard command interface for analog weight programming (analogous to JEDEC's DDR command set):

- SET_TARGET_G(address, G_target, tolerance): Initiate iterative programming to target conductance

- VERIFY_LOOP(max_iterations, convergence_threshold): Configure write-verify parameters

- READ_G_DISTRIBUTION(address_range): Return statistical characterization of programmed states

- CALIBRATE_REFERENCE: Trigger reference column calibration sequence

### 4.2.2 Hardware-Aware Training Interface

Standardize the interface between training frameworks and hardware-aware training (HAT) tools:

- Device Model Format: Define standard JSON/YAML schema for specifying noise, drift, and non-linearity parameters (building on IBM AIHWKit model)

- Noise Injection Protocol: Specify how hardware noise characteristics are translated into training-time perturbations

- Drift Simulation API: Define interface for simulating weight decay during training to ensure deployment robustness

# 5. Safety and Regulatory Alignment

## 5.1 EU AI Act Compliance Framework

The EU AI Act (Article 15) mandates "accuracy, robustness, and cybersecurity" for high-risk AI systems. Standards must provide:

- Robustness Certification: Define test protocols demonstrating consistent performance despite hardware stochasticity and drift

- Technical Documentation Templates: Standardize documentation of drift rates, temperature dependencies, and noise floors as required by Article 11

- HAT as Compliance Tool: Establish Hardware-Aware Training as a recognized method for demonstrating mathematical robustness

## 5.2 ISO 26262 Functional Safety Integration

For automotive deployment, standards must bridge the gap between ISO 26262's deterministic failure model and analog AI's probabilistic behavior:

- Functional Degradation Metrics: Redefine "failure" as accuracy dropping below safety-goal threshold rather than binary device failure

- ASIL Decomposition Templates: Provide reference architectures pairing analog inference (ASIL B/QM) with digital safety monitors (ASIL D)

- Safety Manual Requirements: Define mandatory documentation including Fault Tolerant Time Interval (FTTI) and recalibration frequency

- FMEDA Data Standards: Establish FIT rate reporting format for analog memory devices comparable to IEC 62380 for digital components

## 5.3 Security Standards

Analog AI presents unique security challenges and opportunities:

### 5.3.1 Model Protection

- Side-Channel Mitigation: Define countermeasures against power analysis attacks (SPA/DPA) that exploit current-based computation

- Weight Obfuscation: Standardize address scrambling and encryption protocols for protecting stored weights

### 5.3.2 Physical Unclonable Functions (ISO/IEC 20897)

Leverage inherent device variability for security:

- PUF Quality Metrics: Align with ISO/IEC 20897 requirements for uniqueness, steadiness, and randomness

- Entropy Source Validation: Comply with NIST SP 800-90B for analog noise-based true random number generators

- Fuzzy Extractor Standards: Define error correction requirements for stabilizing noisy PUF responses into cryptographic keys

# 6. Standardization Roadmap

## 6.1 Standards Body Alignment

| Body | Scope | Priority |
| --- | --- | --- |
| JEDEC | Analog memory specifications: endurance, retention, interface commands | HIGH |
| IEEE P2415 | Energy-proportional hardware abstraction for analog accelerators | HIGH |
| IEEE P2851 | Functional safety data exchange for analog reliability modeling | HIGH |
| ISO/IEC JTC1/SC42 | AI system vocabulary and risk management for hardware implementations | MEDIUM |
| UCIe Consortium | Chiplet interface extensions for analog accelerators | MEDIUM |
| NeuroBench | Benchmarking metrics for neuromorphic and analog systems | MEDIUM |

## 6.2 Implementation Timeline

- Phase 1 (2025-2026): Foundation — Establish conductance state specification and drift characterization protocols

- Phase 2 (2026-2027): Interface — Define programming command sets and HAT integration interfaces

- Phase 3 (2027-2028): Compliance — Align with EU AI Act and ISO 26262 requirements; establish certification pathways

- Phase 4 (2028+): Ecosystem — Full NeuroBench integration; 3D architecture standards; photonic/superconducting extensions

# 7. Conclusions and Recommendations

The synthesis of research across analog AI technologies reveals that despite their divergent physics, these systems share fundamental challenges that provide a clear foundation for unified signal standards. The transition from deterministic digital computing to probabilistic analog computing requires a paradigm shift in how we specify, measure, and certify computing hardware.

## 7.1 Key Recommendations

- Adopt Statistical Specifications: Move from deterministic to probabilistic specifications. Define weights as distributions (G_T, σ_G) rather than point values.

- Standardize Drift Management: Establish universal drift characterization protocols and retention classes applicable across PCM, ReRAM, FeFET, and emerging technologies.

- Mandate Differential Encoding: Require G⁺/G⁻ differential pairs as the standard weight representation, providing signed weights and common-mode noise rejection.

- Establish HAT as Compliance Tool: Position Hardware-Aware Training as the bridge between hardware stochasticity and regulatory requirements for accuracy and robustness.

- Engage JEDEC for Analog Memory: Initiate development of JEDEC standards for "Analog Weight Storage" distinct from binary storage standards.

- Adopt NeuroBench Metrics: Transition from MLPerf's binary accuracy thresholds to NeuroBench's energy-aware, event-based metrics for analog system evaluation.

## 7.2 Call to Action

The analog AI ecosystem is at a critical juncture. Without coordinated standardization, the industry risks fragmentation—proprietary interfaces, incompatible calibration protocols, and uncertain regulatory pathways. The common ground identified in this framework provides a foundation for industry-wide collaboration. We call upon semiconductor manufacturers, AI accelerator developers, standards bodies, and regulatory agencies to engage in the development of these essential standards.

The future of energy-efficient AI computing depends on our ability to standardize the analog—transforming physics into reliable, interoperable, and certifiable intelligence.

# References

This framework synthesizes research from the following source categories:

- International Roadmap for Devices and Systems (IRDS) 2024 Edition — Metrology and Advanced Memory Chapters

- NIST Neuromorphic Device Measurements and Physics for Intelligence Programs

- IEEE Working Groups: P2415 (Energy Proportional Systems) and P2851 (Functional Safety)

- ISO/IEC JTC 1/SC 42 AI Standards (22989, 42001, 23894)

- EU AI Act Technical Requirements (Articles 11, 15)

- ISO 26262 Functional Safety Standard

- IBM AIHWKit and Hardware-Aware Training Research

- NeuroBench Consortium Benchmarking Framework

- UCIe 2.0 Specification

- Industry research from IBM, Intel, Innatera, Mythic, SynSense, and academic publications on PCM, ReRAM, FeFET, MRAM, and photonic computing